|  | TRANSMISSION |  | CLUTCH |
| --- | --- | --- | --- |
| - 8mm, 10mm, 27mm, 30mm |  | SOCKETS | - 8mm, 10mm |
| - 10mm, 12mm |  | OPEN END | - 12mm |
| - H5, T30, T40 |  | 3/8 |  |
| - Snap Ring Pliers |  |  |  |
| - 3/8 Ratchet & ext. |  |  |  |
| - 1/2 impact & 3/8 adapt. |  |  |  |
| - Phillips Screwdriver |  |  |  |
| - Pick |  |  |  |
| - Mallet |  |  |  |
| - 1/4 Ext. (to hold oil pump gear) |  |  |  |
| - Channel Locks |  |  |  |
| - Blue Lock Tight |  |  |  |

| Friction Plate Thickness: 2.72 ∼ 2.88 mm (0.107 ∼ 0.113 in.) | Service Limit: 2.5 mm (0.098 in.) |
| --- | --- |
| Clutch Spring Free Length: 48.38 mm (1.905 in.) | Service Limit: 46.3 mm (1.82 in.) |

CLUTCH

| FASTENER | TORQUE | Notes | FASTENER | Torque |
| --- | --- | --- | --- | --- |
| Cutch Hub Nut | 100 ft-lbs | Replace | Front Axle Clamp | 15 ft-lbs |
| Clutch Springs | 78 in-lbs |  | Front Axle | 94 ft-lbs |
| Clutch Cover | 87 in-lbs |  | Rear Axle Nut | 94 ft-lbs |
| Trans Case (M8) | 15 ft-lbs |  | Steering Stem | 58 ft-lbs |
| Trans Case (M6) | 87 in-lbs |  |  |  |
| Shift Pedal Bolt | 18 ft- lbs | Lock Tight |  |  |

# CLUTCH